# Task:	Climbs and Climbing Turns

Definition:	A basic maneuver used to gain altitude while either flying on a constant heading or turning.

Objective:	To develop student skill in maneuvering the airplane after takeoff and while en route to cruising altitude.  Develops student understanding of both pitch and power to result in a gain in altitude and skill in: effect and use of flight controls; turning tendencies; and use of trim; all while using outside references to the horizon.

When:

Where:

Preflight Discussion:

- Effect and use of flight controls.  PITCH + POWER = PERFORMANCE

- Elevator controls pitch by increasing down force.

- Change attitude using pressure on elevator control.

- Airspeed will decrease due to increase in angle of attack.

- Aileron and rudder control for climbing turns.

- Methods to overcome tenseness and over-controlling.

- Visual references for pitch, roll and yaw.

- Nose slightly above horizon  - have student note exact sight picture when correct attitude established.

- Wings equally above horizon (high wing) or equally below horizon (low wing).

- Proper rate of turn for climbing turns

- Exact sight picture of standard rate turn.

- Sight picture will vary to left and right

- Ground references to maintain constant heading.

- Instrument references for pitch, roll and yaw.

- Nose slightly above horizon bar on A/I – have student note exact sight picture when correct attitude established.

- Wings level in relation to horizon bar on A/I.

- Note T/C for proper rate of turn on climbing turns – standard rate – and verify ball is centered.

- For climbs without turns, verify T/C needle is centered.

- Note H/I to check constant heading.

- Establish climb from level cruise flight.

- Back elevator pressure to increase pitch to desired attitude.

- Apply climb power (full throttle) – use friction lock.

- Correct torque effect with rudder pressure.

- Turn ailerons as necessary for climbing turn using standard rate turn.

- Hold backpressure until airspeed stabilizes, and then trim to relieve control pressure.

- Use ailerons to roll out bank.

- Cross check and correct attitude as necessary.

- Level off

- Lead by 30-50 feet (or 10% rate of climb).

- Forward elevator pressure to decrease pitch to level flight attitude.

- When airspeed approaches cruise, reduce power to cruise setting.

- Adjust rudder pressure as torque effect decreases.

- Trim to relieve control pressure when airspeed is stabilized.

- Roll out of climbing turns to headings solely by reference to H/I – lead roll out on H/I by ½ bank angle.

- While decreasing bank, will need to compensate for adverse yaw.

- Turning tendencies

- Engine torque – sustained roll to left – correct with aileron pressure.

- Propeller gyroscopic precession – transient yaw to right – minimal with smooth pitch increase.

- P-Factor – sustained yaw to left during climb – correct with right rudder pressure.

- Slipstream – sustained yaw to left during climb – correct with right rudder pressure.

- Trim technique

- Trim used to relieve control pressure.

- Trim maintains an airspeed (angle of attack), not an altitude.

Instructor Demonstration:

- Establish climb attitude, power and trim.

- Have student describe attitude sight picture when established.

- Have student describe power setting used.

- Have student describe torque correction.

- Have student describe trim technique.

- Level off

- Have student describe attitude sight picture when established.

- Have student describe power setting used.

- Have student describe torque correction.

- Have student describe trim technique.

Student Practice:

- Use integrated technique (IR then VR or VR then IR).

- Use verbal correction – avoid control inputs.

- Have student explain actions during initial performance.

- Watch for errors – correct immediately.

- Attitude control

- Control input sequence – pitch, power, trim.

- Inadequate crosscheck and failure to correctly interpret outside and instrument references.

- Over concentration on pitch – neglecting roll and yaw.

- Incorrect torque correction – too little, ball right; too much, ball left.

- Improper turn coordination on turning climbs – ball to left or right.

- During turn entry

- Exiting turn

- Improper trim technique

- Failure to wait for airspeed to stabilize.

- Not fully relieving control pressure.

- Tenseness and overcontrolling

- Improper trim technique – death grip on wheel.

- Moving controls rather than applying pressures.

- Review performance

- Save critique until performance complete.

- Have student repeat as necessary.

Evaluation:

- Airspeed within +/- 10 knots (Private).

- Heading constant within +/- 20 degrees.

- Altitude within +/- 200 feet.

- Smooth coordinated use of flight controls and power.

- Proper integrated approach; use of both visual references and instrument crosschecks.